T3

Tier 3 · Enterprise Intelligence & Trust

Technical Specification

Storage Architecture, Analytics Algorithms, and Security Design

| Version | 1.0 |
| --- | --- |
| Status | PRODUCTION |
| Zone | IT Zone |
| Standards | InfluxDB 2.x / ISO 22400 OEE / CEF SIEM / OAuth2 RBAC |
| Block Count | 5 blocks |
| Document Date | May 2026 |

# 1. Technical Overview

Tier 3 operates in the IT zone and provides three technical capabilities: durable historian writes (Historian Proxy), statistical operational analytics (OEE Calculator, Trend & Anomaly Detection), and security enforcement (Audit Logger, IAM/RBAC Engine). Five blocks implement these capabilities with zero dependency on raw field data.

## 1.1 Storage Architecture

T1 Digital Twin Sync → publishes to: twins/state-changes (MQTT topic) ↓ T3 Historian Proxy → subscribes to: twins/state-changes → deduplication cache (sliding window per tag) → batches of batchSize points → writes to: InfluxDB 2.x (time-series, measurement = "tag_values") PostgreSQL (relational, table = "tag_values")

## 1.2 Analytics Pipeline

Historian Proxy (write confirmed) → triggers OEE aggregation window OEE Calculator: window.close() → { availability, performance, quality, oee } → Port 1: OEE record (all downstream analytics dashboards) → Port 2: OEE alert if oee < oeeThreshold Tag events (from DTS): → Trend & Anomaly Detection rolling window → Port 1: Normal (stats injected into msg.stats) → Port 2: Anomaly event → T2 Action Logic Engine

# 2. Block Specifications

## 2.1 Historian Proxy

| ID | historian-proxy |
| --- | --- |
| Type | function + influxdb + postgresql |
| Standard | InfluxDB 2.x line protocol / PostgreSQL COPY |
| Dedup algorithm | Sliding window cache: Map<twinId:semanticId, { value, ts }>. Suppresses writes where value unchanged within window. |
| Batch flush | Triggered by batchSize count OR flushIntervalMs timer, whichever fires first |
| InfluxDB line protocol | measurement,assetId=…,semanticId=… value=…,prev=… <nanosecond_timestamp> |
| PostgreSQL insert | INSERT INTO tag_values(asset_id, semantic_id, value, prev, ts) VALUES(…) |
| Output 1 | { written: N, latencyMs: number, target: "influx¦pg¦both" } |
| Output 2 | { error, retryCount, batch: Point[] } — batch preserved for retry |

## 2.2 OEE Calculator

| ID | oee-calculator |
| --- | --- |
| Type | function + aggregator |
| Standard | ISA-95 Part 3 / ISO 22400-2 KPI definitions |
| Formula | OEE = A × P × Q where A = (PPT - DT) / PPT, P = AO / TO, Q = (TP - DC) / TP |
| PPT | plannedProductionTime (seconds per shift, configurable) |
| Window type | Tumbling window — non-overlapping, fixed windowSec duration |
| Output precision | 4 decimal places (toFixed(4)) |
| Alert threshold | oeeThreshold (default 0.75 = 75%). Port 2 fires when oee < threshold. |

## 2.3 Trend & Anomaly Detection

| ID | trend-anomaly |
| --- | --- |
| Type | function + ml-inference |
| Standard | Statistical Process Control (SPC) — Z-Score, IQR, OLS regression |
| Z-Score formula | z = ¦x - μ¦ / σ where μ and σ are computed over windowSize samples |
| IQR formula | outlier if x < Q1 - 1.5*IQR or x > Q3 + 1.5*IQR |
| Trend | Ordinary Least Squares slope: b = Σ(xi - x̄)(yi - ȳ) / Σ(xi - x̄)² |
| Window | Rolling FIFO buffer per tagId, length = windowSize (default 60 samples) |
| Warm-up | No anomaly checks until window contains ≥ minDataPoints samples |
| Output 1 | msg.stats = { mean, std, zScore, trend, algorithm } |
| Output 2 | { ...msg, isAnomaly: true, zScore, tagId } — feeds T2 Action Logic Engine |

## 2.4 Audit Logger

| ID | audit-logger |
| --- | --- |
| Type | function + mqtt-out + file |
| Standard | CEF 0.1 (ArcSight Common Event Format) — SIEM-compatible |
| CEF structure | CEF:Version¦Vendor¦Product¦Version¦SignatureId¦Name¦Severity¦Extension |
| Extension fields | src, act, outcome, msg, rt (receive time epoch ms), cs1 (chain hash) |
| Chain integrity | cs1 = SHA-256(lastHash + currentCEFLine). Tamper detection via broken hash chain. |
| Severity mapping | info→3, warn→5, error→7, critical→8 (configurable via severityMap) |
| Log rotation | Daily rotation when rotateDaily=true. Filename: audit.log.YYYY-MM-DD |
| Output 1 | CEF string published to auditTopic + written to logPath |
| Output 2 | { error, cefLine } on file write or MQTT publish failure |

## 2.5 IAM / RBAC Engine

| ID | iam-rbac |
| --- | --- |
| Type | function + http-request |
| Standard | RBAC (NIST SP 800-162) / OAuth2 (RFC 6749) / LDAP (RFC 4511) |
| Token validation | jwt.verify(token, jwtSecret) — HS256 by default; RS256 supported with publicKey config |
| ACL evaluation | micromatch.isMatch(resource, pattern) for MQTT wildcard matching (+ and #) |
| LDAP integration | When provider=ldap, ldapjs resolves userId → role from configured DN |
| defaultDeny | When true, any unmatched ACL entry results in denial (fail-closed security model) |
| Output 1 | msg.auth = { userId, role } — passes through authorized msg unchanged |
| Output 2 | { userId, role, resource, action, deny: reason } — forwarded to Audit Logger |

# 3. Configuration Reference

| Block | Parameter | Type | Default / Notes |
| --- | --- | --- | --- |
| Block | Parameter | Type | Default / Notes |
| Historian | influxUrl | url | http://influx:8086 |
| Historian | influxBucket | string | factory |
| Historian | batchSize | number | 100 points |
| Historian | flushIntervalMs | number | 5000ms |
| Historian | deduplicateWindow | string | 1s (parsed to ms) |
| OEE | windowSec | number | 300 (5 minutes) |
| OEE | oeeThreshold | number | 0.75 (75%) |
| OEE | plannedProductionTime | number | 28800 (8h shift in seconds) |
| Anomaly | algorithm | enum | z-score ¦ iqr ¦ trend |
| Anomaly | zThreshold | number | 3.0 standard deviations |
| Anomaly | windowSize | number | 60 samples |
| Anomaly | minDataPoints | number | 30 samples (warm-up) |
| Audit | cefVersion | string | 0 |
| Audit | severityMap | string | info:3,warn:5,critical:8 |
| Audit | rotateDaily | bool | true |
| RBAC | provider | enum | local ¦ ldap ¦ ad |
| RBAC | defaultDeny | bool | true (fail-closed) |
| RBAC | auditDenials | bool | true (forward to Audit Logger) |

# 4. Security Architecture

## 4.1 Defense-in-Depth

T3 implements two security controls that operate at different layers. The IAM/RBAC Engine is a preventive control — it blocks unauthorized requests before they reach sensitive blocks. The Audit Logger is a detective control — it records what happened for post-incident forensic analysis.

| Preventive layer | IAM/RBAC Engine — fail-closed (defaultDeny=true). All requests denied unless explicitly permitted. |
| --- | --- |
| Detective layer | Audit Logger — CEF events with SHA-256 chain integrity. Any gap in the hash chain indicates log tampering. |
| Key management | jwtSecret should be stored in Node-RED credentials (encrypted) not in flow properties. |
| LDAP/AD integration | Use LDAPS (port 636) or STARTTLS for LDAP connections in production. |

## 4.2 Data Retention

InfluxDB buckets should have a configured retention policy appropriate for the use case. Audit logs should be retained for a minimum of 90 days for regulatory compliance. PostgreSQL historian data can be archived to cold storage using pg_dump on a scheduled basis.

## 4.3 Integration with T2 Action Logic Engine

T3 Anomaly Detection (Port 2) feeds directly into T2 Action Logic Engine. This integration creates the closed-loop feedback path. The technical contract is:

// T3 Anomaly Detection Port 2 output → T2 ALE input { payload: { impactArray: string[], // from prior Asset Graph BFS query triggerType: "anomaly", triggerValue: number, // the anomalous value tagId: string, zScore: number, isAnomaly: true, assetContext: { ... } // full T1 context preserved }, stats: { mean, std, zScore, trend, algorithm } }